SNA Series

Overview

The SNA series capacitors with double side metallized

carrier film with internal series connection and metalized

polypropylene film, polyester wrapping with epoxy

resin fill.

Application:

The SNA series is designed for snubber/pulse applications.

For high pulse and high frequency applications requiring

extremely reliable contacts e.g. IGBT-applications

Benefits

Self-healing

Very low dissipation factor

Very high ripple current

Internal series connection

Negative capacitance change versus temperature

Construction

Dielectric: Polypropylene (PP) film.

Capacitor electrodes: Double-sided metallized plastic film.

Protection: Polyester wrapping with epoxy resin fill.

Terminals: Tinned copper wire (lead-free).

Internal construction:

Structure of ordering code

SNA – 1200 - 0.47 – 44  D

1     2   3    4   5

1  Series code

2  Rated voltage: 1200 V

3  Rated capacitance: 0.47 μF

4  Length of capacitor

5  D for round axial, F for flat oval

SNA Series

Reference standards                         IEC 61071 , IEC 60068 , RoHS compliance.

Degree of protection                         IP00

Rated capacitance (CN)                       0.047μF ... 6.8μF

Rated voltage (UNDC)                         700V ... 3000V

Standard capacitance tolerance                 K: ±10% , J: ±5%

Dissipation factor tan δ (1kHz@20°C)             ≤ 5.0 • 10-4

Test voltage between terminals UTT               1.5 UNDC , 10s

Test voltage between terminals and case UTC        4000 VAC, 10s

Insulation Resistance                      30000s but need not exceed 30GW (typical value),

after 1 minute of electrification at 100Vdc (25 ± 5°C).

Self inductance                        ≤1nH/mm of capacitor and leads length used for connection

Operating temperature range (case)          –40 °C ... +105°C

Storage temperature Θstg                 –40 °C ... +105 °C

Climatic category                        40/105/56

Capacitance deviation                   in the operating Temperature range of -40 to +85 °C,±1.5%

max on capacitance value measured at +20 °C

Expected lifetime                       100 000 h at UNDC @ Θhs 85°C

Capacitance drop at end of life              -3% (typical)

Fit rate                               50 (100 000 h at UNDC @ at Θhs 85°C)

SNA  Designs

SNA Series

UN  1600V      Us  2400V      Urms  550V

| Ordering code | Cn (μF) | DIMENSIONS (mm) |  |  |  |  |  | du/dt (V/μs) | Ipk (A) | Rs @100kHz (mΩ) | Imax 20kHz@70° (A) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Designs D |  | Designs F |  |  | d |  |  |  |  |
|  |  | D | L | W | T | L |  |  |  |  |  |
| SNA-1600-0.10-32D | 0.10 | 14.5 | 32 |  |  |  | 1.0 | 1500 | 150 | 7.1 | 7 |
| SNA-1600-0.10-32F | 0.10 |  |  | 18.0 | 10.0 | 32 | 1.0 | 1500 | 150 | 7.1 | 7 |
| SNA-1600-0.15-32D | 0.15 | 17.5 | 32 |  |  |  | 1.0 | 1500 | 225 | 5.0 | 9 |
| SNA-1600-0.15-32F | 0.15 |  |  | 20.5 | 12.5 | 32 | 1.0 | 1500 | 225 | 5.0 | 9 |
| SNA-1600-0.22-44D | 0.22 | 16.5 | 44 |  |  |  | 1.0 | 1030 | 227 | 6.8 | 9 |
| SNA-1600-0.22-44F | 0.22 |  |  | 20.0 | 12.0 | 44 | 1.0 | 1030 | 227 | 6.8 | 9 |
| SNA-1600-0.33-44D | 0.33 | 20.0 | 44 |  |  |  | 1.2 | 1030 | 340 | 4.8 | 12 |
| SNA-1600-0.33-44F | 0.33 |  |  | 23.0 | 15.0 | 44 | 1.2 | 1030 | 340 | 4.8 | 12 |
| SNA-1600-0.47-44D | 0.47 | 23.5 | 44 |  |  |  | 1.2 | 1030 | 484 | 3.6 | 12 |
| SNA-1600-0.47-44F | 0.47 |  |  | 27.5 | 18.0 | 44 | 1.2 | 1030 | 484 | 3.6 | 12 |
| SNA-1600-0.68-57D | 0.68 | 23.0 | 57 |  |  |  | 1.2 | 640 | 435 | 4.4 | 12 |
| SNA-1600-0.68-57F | 0.68 |  |  | 27.0 | 27.5 | 57 | 1.2 | 640 | 435 | 4.4 | 12 |
| SNA-1600-1.0-57D | 1.0 | 28.0 | 57 |  |  |  | 1.2 | 640 | 640 | 3.2 | 12 |
| SNA-1600-1.0-57F | 1.0 |  |  | 32.5 | 21.5 | 57 | 1.2 | 640 | 640 | 3.2 | 12 |
| SNA-1600-1.5-57D | 1.5 | 34.0 | 57 |  |  |  | 1.2 | 640 | 960 | 2.3 | 12 |
| SNA-1600-1.5-57F | 1.5 |  |  | 38.0 | 27.0 | 57 | 1.2 | 640 | 960 | 2.3 | 12 |
| SNA-1600-2.0-57D | 2.0 | 39.0 | 57 |  |  |  | 1.2 | 640 | 1280 | 1.9 | 12 |
| SNA-1600-2.0-57F | 2.0 |  |  | 44.0 | 31.5 | 57 | 1.2 | 640 | 1280 | 1.9 | 12 |

(1) Maximum values at 100kHz, +60°C for case operating T= +85°C

(2) Typical values at 100kHz.